ARCHITECTURE CO-DESIGN FOR A CLOUD INFERENCE ACCELERATOR: PERFORMANCE, MEMORY, PIPELINE AND I/O ANALYSIS

### Assignment Information

| Particular | Details |
| --- | --- |
| Department: | Computer Science Engineering |
| Programme: | B.E. / B.Tech |
| Course Code & Course Name | CSA1223 – Computer Architecture |
| Academic Year / Batch | 2026-2027 |
| Faculty Name | S.Agnes Shifani |
| Assignment Title | Architecture Co-Design for a Cloud Inference Accelerator: Performance, Memory, Pipeline and I/O Analysis |
| Date of Issue | [1-9-2026] |
| Date of Submission | [2-9-2026] |
| Maximum Marks | 100 |
| Course Outcome(s) – CO | CO3, CO4, CO5, CO7, CO8 |
| Bloom's Taxonomy Level | L4 – Analyze; L5 – Evaluate; L6 – Create |
| SDG Mapping (SDG 1-17) | SDG 9 – Industry, Innovation and Infrastructure; SDG 11 – Sustainable Cities and Communities |
| Industry / Societal Relevance | Enterprise networking, branch connectivity, cloud access, voice/video services, QoS, routing resilience, cybersecurity, and efficient use of network resources. |

PROBLEM UNDERSTANDING AND FORMULATION

1. Industry Background

Modern cloud and edge inference systems process large amounts of data from cameras and other sensors. A compact inference appliance must process multiple camera streams simultaneously while maintaining low latency.

In this problem, a technology company needs an inference appliance capable of handling four simultaneous camera streams. Each stream generates approximately 12 MB/s of pre-processed feature data.

Therefore, the total feature-data input requirement is:

4×12=48 MB/s

48 MB/s

The system also performs integer calculations, fixed-point multiplication and accumulation, floating-point normalization, branching and periodic model-parameter transfers from storage.

2. Engineering Problem

The baseline processor operates at:

- Clock frequency = 2.4 GHz

- Ideal CPI = 1.0

- Workload = 1 million instructions

Instruction distribution:

| Instruction Type | Percentage | Number of Instructions |
| --- | --- | --- |
| Integer ALU | 45% | 450,000 |
| Memory | 15% | 150,000 |
| Floating-point | 20% | 200,000 |
| Branch | 10% | 100,000 |
| Control/Other | 10% | 100,000 |
| Total | 100% | 1,000,000 |

The architecture must achieve:

Average inference latency<2.0 ms

Input bandwidth≥48 MB/s

The assignment specifically requires controlled cache misses, hazards, arithmetic workloads and I/O contention to be introduced during evaluation.

3. Stakeholders

The major stakeholders are:

- Technology company – requires a reliable and efficient inference appliance.

- System designers – design the processor, memory and I/O architecture.

- AI/ML application developers – require low inference latency.

- Camera/data sources – continuously generate feature data.

- Cloud/data-centre infrastructure – provides model parameters and storage.

- End users – expect fast and reliable camera-based inference.

4. Technical Requirements

The proposed architecture must:

- Process four simultaneous camera streams.

- Sustain at least 48 MB/s input.

- Execute integer operations.

- Perform fixed-point MAC operations.

- Perform floating-point normalization.

- Support branch/control operations.

- Transfer model parameters from storage.

- Minimize pipeline stalls.

- Reduce cache miss penalties.

- Support DMA-based data movement.

- Handle I/O contention.

- Achieve less than 2 ms average inference latency.

BASELINE PERFORMANCE ANALYSIS

1. Number of Clock Cycles

Given:

IC=1,000,000
CPI=1.0

Therefore,

Cycles=IC×CPI
=1,000,000×1
1,000,000 cycles

2. Clock Period

Processor frequency:

f=2.4GHz

Therefore:

T=12.4×109
T=0.4167ns

3. Execution Time

Execution Time=CyclesClock Frequency
=1,000,0002.4×109
=0.0004167s

Therefore:

Execution Time=0.4167ms

Target Validation

Required:

T<2ms

Baseline:

T=0.4167ms

Therefore:

0.4167ms<2ms

So, the ideal baseline processor satisfies the latency target for the representative 1-million-instruction workload.

However, this is an ideal CPI calculation. Real execution introduces cache misses, branch penalties, data hazards, structural hazards and I/O delays. Therefore, actual CPI will be greater than 1.0.

INSTRUCTION MIX AND COMPUTATIONAL WORKLOAD

1. Integer ALU Operations – 45%

Number of integer instructions:

1,000,000×0.45=450,000

These instructions perform:

- Address calculation

- Array indexing

- Pointer manipulation

- Loop counters

- Integer comparisons

Architectural Requirement

The processor should provide:

- Integer ALU

- Fast register file

- Effective address generation

- Forwarding paths

2. Memory Instructions – 15%

1,000,000×0.15=150,000

These instructions access:

- Input feature data

- Intermediate results

- Model parameters

- Stack/data structures

Memory operations are particularly sensitive to cache misses.

3. Floating-Point Operations – 20%

1,000,000×0.20=200,000

Floating-point operations are used mainly for normalization.

A floating-point unit should therefore support:

- Addition

- Multiplication

- Division/reciprocal operations

- Floating-point normalization

4. Branch Instructions – 10%

1,000,000×0.10=100,000

Branches are required for:

- Conditional processing

- Loop execution

- Decision-making

- Model control flow

Branches introduce control hazards, especially in a pipeline.

5. Control/Other Instructions – 10%

1,000,000×0.10=100,000

These include:

- Control operations

- System operations

- Miscellaneous instructions

ARITHMETIC OPERATION DESIGN

The assignment requires integer, fixed-point MAC and floating-point workloads to be       incorporated into the architecture.

1. Integer Address Calculation

For an array:

Address=Base+Index×ElementSize

For example, if:

- Base address = 1000

- Index = 20

- Element size = 4 bytes

Then:

Address=1000+20×4
Address=1080

A dedicated address-generation unit can perform such operations efficiently.

3. Floating-Point Normalization

Floating-point normalization adjusts values into an appropriate numerical range.

A simplified normalization operation can be represented as:

Xnorm=X-μσ

where:

- X= input

- μ= mean

- σ= standard deviation

The floating-point unit performs the arithmetic operations required for normalization.

PROPOSED ARCHITECTURE

Two alternatives are evaluated.

Alternative A – Deep In-Order Pipeline

Alternative A uses:

- Deeper pipeline

- Conventional cache hierarchy

- In-order instruction execution

- DMA-based I/O

A suitable conceptual pipeline is:

Fetch→Decode→Register Read→Execute→Memory→Writeback

Main Features

- Simple control

- Lower hardware complexity

- Lower power/resource requirements

- Conventional L1/L2/L3 cache

- DMA for feature-data transfers

Limitation

When an instruction stalls, subsequent instructions may also be delayed because execution remains in order.

Alternative B – Superscalar Out-of-Order Architecture

Alternative B uses:

- Multiple instruction issue

- Out-of-order execution

- Speculative execution

- Larger cache capacity

- Aggressive memory overlap

- DMA/I/O overlap

Conceptually:

┌── Integer ALU ──────┐

Fetch → Decode → ├── Fixed-Point MAC ──┤ → Writeback

├── Floating Point ───┤

└── Branch Unit ──────┘

↑

Reorder Buffer

↑

Reservation Stations

Main Advantages

Independent instructions can execute while another instruction waits for memory.

This reduces the effect of:

- Cache misses

- Data dependencies

- Branch delays

- Memory latency

The assignment specifically asks for evaluation of out-of-order and speculative execution.

PIPELINE HAZARD ANALYSIS

1. Data Hazard

A data hazard occurs when one instruction depends on the result of another instruction.

Example:

ADD R1, R2, R3

SUB R4, R1, R5

The second instruction requires R1 produced by the first.

Solution

Use:

- Operand forwarding

- Pipeline interlocks

- Instruction scheduling

- Out-of-order execution

2. Control Hazard

Control hazards occur due to branch instructions.

Example:

BEQ R1, R2, LABEL

The processor may not know which instruction should be fetched next until the branch is resolved.

Solution

- Branch prediction

- Speculative execution

- Early branch resolution

Alternative B has a major advantage here because it supports speculative execution.

3. Structural Hazard

A structural hazard occurs when multiple instructions require the same hardware resource simultaneously.

Example:

Two instructions require the same memory port.

Solution

- Multiple execution units

- Separate instruction/data caches

- Multiple memory ports

- Resource scheduling

PIPELINE STALL MODEL

For a realistic model, controlled stalls can be introduced.

Assume:

- Base CPI = 1.0

- Data-hazard penalty = 0.08

- Control-hazard penalty = 0.12

- Cache-miss penalty = 0.15

- I/O contention penalty = 0.05

Then:

CPIA=1+0.08+0.12+0.15+0.05
CPIA=1.40

Therefore:

CyclesA=1,000,000×1.40
1.4×106 cycles

Execution time:

TA=1.4×1062.4×109
0.5833ms

This remains below 2 ms for the assumed workload.

Important: These penalty values are proposed modelling assumptions for simulation. They are not given numerical values in the original assignment. The assignment itself requires controlled hazards and cache misses to be introduced.

Proposed Cache Organization

| Level | Proposed Role | Main Purpose |
| --- | --- | --- |
| L1 | Small and fastest | Frequently used instructions/data |
| L2 | Medium | Reduce DRAM accesses |
| L3 | Larger shared cache | Shared model/intermediate data |
| DRAM | Main memory | Large feature/model data |
| SSD/Storage | Persistent storage | Model parameters |

MEMORY BANDWIDTH ANALYSIS

Four camera streams generate:

4×12=48MB/s

Therefore:

Required Bandwidth=48MB/s

If the designed memory path supports, for example:

B=100MB/s

then utilization is:

Utilization=48100×100
48%

Therefore, the memory system has sufficient bandwidth under the assumed 100 MB/s effective transfer capacity.

I/O AND DMA DESIGN

DMA is used to transfer camera feature data directly between an I/O device and memory without requiring the CPU to execute an instruction for every transferred byte.

Proposed Data Path

Camera 1 ─┐

Camera 2 ─┤

Camera 3 ─┼──→ DMA Controller ──→ RAM

Camera 4 ─┘              ↑

|

CPU

Each camera generates:

12MB/s

Four cameras:

48MB/s

The DMA controller should therefore sustain at least:

48MB/s

I/O CONTENTION TEST

To test I/O contention:

Normal Condition

Four streams:

48MB/s

Contention Condition

Add periodic model-parameter transfers from storage.

The shared memory/bus may then become busy.

Expected effect:

- Increased memory latency

- Increased DMA waiting

- Increased CPU stalls

- Increased CPI

- Increased inference latency

ALTERNATIVE A PERFORMANCE

For modelling purposes, assume the following realistic CPI contributions:

| Factor | CPI Contribution |
| --- | --- |
| Ideal CPI | 1.00 |
| Data hazards | 0.08 |
| Control hazards | 0.12 |
| Cache misses | 0.15 |
| I/O contention | 0.05 |
| Total CPI | 1.40 |

CPIA=1.40

Execution time:

TA=0.5833ms

ALTERNATIVE B PERFORMANCE

| Factor | CPI Contribution |
| --- | --- |
| Ideal CPI | 1.00 |
| Data hazards | 0.03 |
| Control hazards | 0.04 |
| Cache misses | 0.07 |
| I/O contention | 0.03 |
| Estimated CPI | 1.17 |

Therefore:

CPIB=1.17

Cycles:

1,000,000×1.17
=1,170,000 cycles

Execution time:

TB=1,170,0002.4×109
TB=0.4875ms

ALTERNATIVE COMPARISON

| Parameter | Alternative A | Alternative B |
| --- | --- | --- |
| Architecture | In-order | Superscalar OoO |
| Base frequency | 2.4 GHz | 2.4 GHz |
| Assumed CPI | 1.40 | 1.17 |
| Instructions | 1 million | 1 million |
| Cycles | 1.40 M | 1.17 M |
| Execution time | 0.5833 ms | 0.4875 ms |
| Cache | Conventional | Larger |
| Branch handling | Basic prediction | Speculative execution |
| Memory overlap | Moderate | High |
| DMA | Supported | Supported |
| Hardware complexity | Low | High |
| Resource usage | Lower | Higher |
| Scalability | Moderate | High |
| Performance | Good | Better |
| Expected latency target | Met | Met |

The assignment expects the alternatives to be compared using CPI, latency, stalls, cache behaviour, bandwidth, complexity, scalability and resource efficiency.

WHY ALTERNATIVE B IS RECOMMENDED

Alternative B is recommended because the application contains a combination of:

- High-volume memory operations

- Floating-point operations

- Fixed-point MAC operations

- Branch instructions

- Continuous camera input

- Periodic model transfers

Out-of-order execution allows independent instructions to continue executing while another instruction waits for memory.

The larger cache reduces the probability of expensive memory accesses.

Speculative execution reduces the effect of branch hazards.

DMA allows camera data to move into memory without continuous CPU intervention.

Therefore, Alternative B provides better overall performance and scalability, although it requires greater hardware complexity and resource consumption.

SIMULATION / MODEL IMPLEMENTATION

The assignment requires implementation and results, including pseudocode and GitHub upload.

A Python-based architectural performance model can be used to simulate the two alternatives.

Simulation Inputs

Clock frequency = 2.4 GHz

Instructions = 1,000,000

Ideal CPI = 1.0

Integer = 45%

Memory = 15%

Floating point = 20%

Branch = 10%

Control/Other = 10%

Camera streams = 4

Data/stream = 12 MB/s

Required bandwidth = 48 MB/s

Latency target = 2 ms

PSEUDDECODE

START

Set clock_frequency = 2.4 GHz

Set instruction_count = 1,000,000

Set ideal_CPI = 1.0

Calculate:

integer_instructions = 45%

memory_instructions = 15%

floating_point_instructions = 20%

branch_instructions = 10%

control_instructions = 10%

Calculate camera_bandwidth:

total_bandwidth = 4 × 12 MB/s

For each architecture:

Calculate base_cycles

Add data hazard stalls

Add control hazard stalls

Add cache miss stalls

Add I/O contention stalls

Calculate effective CPI

Calculate total cycles

Calculate execution time

Check:

execution_time < 2 ms

Check:

bandwidth >= 48 MB/s

Compare:

CPI

Execution time

Pipeline stalls

Cache effects

Memory utilization

I/O behaviour

Complexity

Scalability

Select better architecture

END

TEST CASES AND VALIDATION

The second document recommends validating a design through normal-load testing, congestion testing, failure testing and measurable performance metrics.

For this Computer Architecture assignment, the following test cases can be used.

Test Case 1 – Ideal Workload

| Parameter | Value |
| --- | --- |
| Instructions | 1,000,000 |
| CPI | 1.0 |
| Frequency | 2.4 GHz |
| Execution time | 0.4167 ms |
| Result | PASS |

Test Case 2 – Pipeline Hazards

Introduce:

- Data dependencies

- Branches

- Resource conflicts

Expected: CPI>1

Result:

- Increased execution time

- Increased pipeline stalls

Test Case 3 – Cache Misses

Increase L1/L2 cache misses.

Expected:

- Higher memory latency

- Higher CPI

- Higher execution time

Test Case 4 – I/O Contention

Run four camera streams simultaneously and periodically transfer model parameters.

Required:

Bandwidth≥48MB/s

Expected result:

48MB/s or greater

Test Case 5 – Full Workload

Combine:

- Integer calculations

- MAC operations

- Floating-point normalization

- Branches

- Cache misses

- DMA transfers

- I/O contention

The final architecture should satisfy:

Latency<2ms

Bandwidth≥48MB/s

EXPECTED RESULTS

An example model-based result table is:

| Metric | Alternative A | Alternative B |
| --- | --- | --- |
| Effective CPI | 1.40 | 1.17 |
| Execution time | 0.5833 ms | 0.4875 ms |
| Required input bandwidth | 48 MB/s | 48 MB/s |
| Target latency | <2 ms | <2 ms |
| Cache performance | Moderate | Better |
| Pipeline stalls | Higher | Lower |
| Memory overlap | Moderate | High |
| I/O overlap | Good | Better |
| Complexity | Lower | Higher |
| Scalability | Moderate | High |
| Final decision | Suitable | Recommended |

Alternative A

Advantages

- Simple design

- Lower hardware complexity

- Easier implementation

- Lower resource requirements

- Easier debugging

Disadvantages

- More pipeline stalls

- Lower instruction-level parallelism

- More sensitive to cache misses

- Lower scalability

Alternative B

Advantages

- Higher instruction-level parallelism

- Lower effective stall impact

- Better cache utilization

- Better memory overlap

- Better branch handling

- Better scalability

Disadvantages

- More complex hardware

- Higher resource consumption

- More difficult implementation

- Greater power/area requirements

BROADER INDUSTRY CONSIDERATIONS

1. Reliability

The system must continuously process four camera streams. Failure of the processing or memory subsystem can interrupt inference.

Therefore, reliable DMA operation, memory protection and error handling are important.

2. Scalability

The architecture should be capable of supporting additional camera streams or larger models in the future.Alternative B provides better scalability because it can exploit higher instruction-level and memory-level parallelism.

3. Resource Efficiency

Alternative A consumes fewer resources, while Alternative B uses additional hardware for:

- Multiple execution units

- Reorder buffer

- Reservation stations

- Larger caches

- Branch prediction

Therefore, the final choice depends on the required performance and available hardware resources.

ENGINEERING DECISION

Based on the calculated performance:

CPIA=1.40
CPIB=1.17

and:

TA=0.5833ms
TB=0.4875ms

Alternative B provides approximately:

0.5833-0.48750.5833×100
16.4%

lower execution time than Alternative A under the assumed stall model.

CONCLUSION

The architecture co-design problem was analyzed for a compact inference appliance supporting four simultaneous camera streams. Each stream produces 12 MB/s, resulting in an aggregate feature-data requirement of 48 MB/s.

The representative workload consists of 45% integer ALU instructions, 15% memory instructions, 20% floating-point instructions, 10% branch instructions and 10% control/other instructions.

The ideal baseline processor at 2.4 GHz and CPI 1.0 requires:

0.4167ms

for one million instructions.

The design satisfies the required targets of:

Latency<2.0ms

Aggregate input bandwidth≥48MB/s

subject to validation through the actual simulation/model.

Expected Outcome

The proposed architecture is expected to:

- Achieve < 2 ms inference latency.

- Sustain ≥ 48 MB/s camera-data input.

- Reduce pipeline stalls and cache-miss delays.

- Efficiently perform integer, MAC, and floating-point operations.

- Enable efficient DMA-based data transfer.

Compare Alternatives A and B and select the better-performing architecture

FINAL SUMMARY

| Requirement | Result |
| --- | --- |
| Four camera streams | Supported |
| Data per stream | 12 MB/s |
| Aggregate input | 48 MB/s |
| Processor frequency | 2.4 GHz |
| Baseline CPI | 1.0 |
| Baseline execution time | 0.4167 ms / 1M instructions |
| Alternative A | In-order + conventional cache + DMA |
| Alternative B | Superscalar + OoO + speculative + larger cache |
| Recommended architecture | Alternative B |
| Latency target | < 2.0 ms |
| Bandwidth target | ≥ 48 MB/s |
| DMA | Required and included |
| Cache-miss testing | Included |
| Pipeline hazard testing | Included |
| I/O contention testing | Included |
| SDG 9 | Aligned |
| SDG 12 | Aligned |

Final result :